| Model Name | ESW-3128 | Version | 1.00.0A | Designer | Approval |
| --- | --- | --- | --- | --- | --- |
| Part Name | Firmware | Date | 2017/04/18 |  |  |
| Part Number | N/A | Unit | N/A |  |  |
| File Name | ESW-3128_Release note_1.00.0A_20170418.doc |  |  |  |  |

Release Note

Firmware Version 1.00.0A	2

Firmware Version 1.00.09	3

Firmware Version 1.00.08	4

Firmware Version 1.00.07	7

Firmware Version 1.00.06	7

Firmware Version 1.00.05	8

Firmware Version 1.00.04	9

Firmware Version 1.00.03	9

Firmware Version 1.00.02	9

Firmware Version 1.00.01	10

# Firmware Version 1.00.0A

Date:  2017/04/18

| New Feature / System Release | Description |
| --- | --- |
| System | Added that the time out can support the range from 1 to 1440 and the time (sec, minute) for console, SSH, Telnet. |
| System Service | SNMP trap & syslog can support CPU load. |
| Syslog | Support more detail only on syslog such as STP/LACP status change. |
| Port Counter | Port counter allow to clear per port counter. |
| Syslog Configuration | Add new feature / User log function. (Enable/Disable record user keying CLI commands) CLI commands: syslog logging-type terminal-history. no syslog logging-type terminal-history. |
| Update | Description |
| WEB | Modified that the time zone replaces GMT with UTC. |
| User log | Fixed that it can’t show IP address of user on ssh mode issue. |
| VLAN QinQ | Fixed that it can’t support double tag on packet when enable QinQ mode with ether type is 8100. |
| DHCP snooping | Fixed that it will crash when it receives a DHCP offer with a 9100 single vlan tag. |
| DHCP snooping | Fixed that it will crash when enabled DHCP snooping and server trust port receiving DHCPv6 packet with empty value. |
| SSH | Fixed that when using Putty or SecureCRT to access device via SSH in the first time will see "login as" then change to "Username" issue. |
| SSH | Modified that the user login count limitation to 3. |
| SSH | Fixed that it can’t work fine issue. |
| DHCPv6 | Fixed that when client port receives DHCPv6 confirm packet, it always will not forward to server trust port. |
| User Authentication | Modify all UI about MD5 hashed password title/command/desc. |
| Load Factory Settings | Fixed that when execute “Load Factory Settings Except Network Configuration” will resend dhcpv6 Solicit packet. |
| SNMP | Fixed that if port is link down but use SNMP to show port status also have speed value, after modified it is display zero now. |
| LLDP | Fixed that when enable lldp port config before lldp initial delay complete, should not cause first lldp packet sent out after 2 min ; LLDP packet will send out in 5 secs now. |
| User Authentication | Fixed that user can to see the other user’s password in “User Authentication” in html. |
| DHCP | Fixed that when receive double tag DHCP packet from Server Trust port(snooping enable/QinQ disable) then DHCP Snooping program will be die. |
| CLI | Fixed that when enter"show switch-info cpu-mem-statistics" display wrong title with "Statistics". |
| WEB | Fixed that if the Static MAC Table is Broadcast MAC address then will pop message but pop wrong word "Broadcast". |

# Firmware Version 1.00.09

Date:  2016/09/30

| New Feature / System Release | Description |
| --- | --- |
| System | If config partition (Cimage) damage(or not completely) in system bootup lead to DUT can’t access then will auto recovery config data |
| CLI / Terminal length | CLI add new commands for show show running-config、show default-config、show start-up-config can specify length: terminal length [0-512] <cr> no terminal length <cr> show terminal <cr> |
| MAC Table Address | CLI add show MAC table with sorting and counting functions: Add: show mac address-table interface [port] <cr> show mac address-table mac [mac-addr] <cr> show mac address-table vlan [vlan_id] <cr> Remove: show mac address-table top <cr> Keep: show mac address-table all <cr> |
| Firmware Upgrade | If firmware upgrade fail ( kernel partition damage) then check that image version number on UI after DUT bootup in another image, it should be null. |
| Security / PPPoE | PPPoE server trust ports |
| Update | Description |
| DHCP Snooping | Modify Security Configuration – “Filter configuration” change name to “DHCP snooping” |
| Syslog | Modify “Mal-attempt Log configuration” change name to “Syslog Configuration” |
| IGMP Snooping | Modify WEB UI IGMP snooping title "IGMP/MLD snooping v3" --> "IGMPv3 / MLD v2 snooping". |
| RSTP | Modify RSTP default configure: - System Priority from 4096 to 32768 - Port Priority from 0 to 128 |
| System information | Modify UI show switch information about firmware version title Current Boot Image Configured Boot Image Image-1 Version Image-2 Version |
| ACL / CLI | CLI add acl command “action permit” |
| ACL / CLI | CLI can't set command "acl ingress-port any" successfaully. |
| DHCP Snooping / Option82 / CLI | CLI set option 82 formated commands is “ip dhcp snooping remote formatted” instead of “ip dhcp snooping formatted” |
| CLI | CLI remove "quit" command. |
| SNMP / MIB | Fixed that SNMP IF-MIB (ifDescr / iName / ifAlias) display incorrect info |
| Port Status | Fixed that CLI support show port description but WEB UI "port status" page not show port description |
| IP Source Binding | Fixed that after enable IP source binding then we cannot access DUT from front console port |
| Image info. | When do "show running-config" or "backup config", title of FW version always display image-1 version, even current is image-2. |
| DHCP | Fixed that device will send 2 different DHCP session after restore configure |
| Syslog | Fixed that DUT can’t send syslog packet |
| DHCP Snooping | Fixed that DHCP packet can't forward to DHCP clients when enabled DHCP snooping |

# Firmware Version 1.00.08

Date:  2016/05/30

| New Feature / System Release | Description |
| --- | --- |
| System | Rewrite the process to avoid the issues of linux system or busybox |
| System | Auto-reboot system in case of out of memory suddenly |
| MAC limit | MAC limit per port in WEB, CLI and SNMP |
| IGMPv3 | Enable or Disable IGMPv3 function |
| DHCP Option 82 | DHCP OPT 82 Formatted function and related CLI |
| TACACS+ | Support user authentication “TACACS+” |
| User authentication method | Support user authentication method MD5 hashed password function |
| Update | Description |
| SNMP / trap | Power down trap(dying_gasp) send trap frequency from 3times/1min to 3times/1sec |
| Firmware Upgrade | Restricted FOS firmware upgrade can only to a corrupted partition if it exists |
| IGMP Snooping | When IGMP Query interval is less than Query Response interval , after reboot or restore config it will make IGMP query interval value change. |
| IGMP Snooping | Modify IGMP query response interval parameter config range: From 1-6000 change to 1-255 |
| IGMP Snooping | Fixed IGMP snooping table not clear under Query interval less than response interval. |
| VLAN QinQ | CLI add commads about VLAN QinQ |
| LLDP | CLI modified show LLDP information title |
| ACL | CLI add new command “acl action deny” |
| ACL | Change CLI command “acl no action” config from “deny” to “permit” |
| ACL | Remove CLI command “acl action permit” |
| ACL | Modify CLI show configure |
| DHCP Option 82 Remote-ID | Fixed the wrong packet content for remote id user manual string. |
| SSH | SSH function can’t not work. |
| DHCP Option 82 Circuit-ID | When default value with Option82_Circuit-ID, the packet of first byte should be display "00", but it display "01" |
| IGMP Snooping | Fixed that IPTV will freeze and IGMP table sudently empty |
| DHCP Option 82 Remote-ID | Fixed that WEB can’t show default DHCP OPT82 remote-id with MAC address |
| DHCP | After change vender id DUT not send DHCP release packet. |
| IGMP Snooping | Multicast stream lost while client join/leave to same group lead to IPTV freezes |
| SNMP / Event log | SNMP can not display the IPv4 address of the login user in the event log table correctly |
| Restore configure / IGMP | After restore config igmp vlan id config would be set to default value even the igmp vlan id config is empty |
| IGMP Snooping | Fix the issue that igmp snooping receive igmpv3 packet with illegal value (number of group record) cause program crash. |
| IGMP Snooping | Fix issue that group record number in igmpv3 packet be changed by igmp snooping module |
| SNMP / Trap | When DUT is GB version but SNMP trap object ID is same as CTS version |
| DHCP Snooping | Switch a port from unlimit to DHCP of IP sourceguard. Then it will drop DHCPv6 renew packets until it receives a DHCPv6 solicit |
| MAC Address Table | When all UI set same MAC address and same VID at different port in Static MAC Address Table, system not pop warning message even setting not success |
| MAC Address Table | CLI/WEB/SNMP can’t set broadcast MAC address in Static MAC table now |
| Static IP Table | The entry in the Static IP Table Configuration can not be deleted by the telnet command |
| DHCP Snooping | SNMPc can not add IPv4 address into DHCP Snooping table successfully. And the original IPv6 address doesn't disappear and will be transformed |
| SNMP / NTP | Check from the SNMP server. The "SNTP status" is "Enabled". Check from web and telnet. The SNTP status is "Disabled" |
| 802.1X | The 802.1X statistics table can not be updated |
| System / RSTP | Plug out one forward port and plug in again, than sent 1 pps forwarding packet from other port, RSTP port will can't blocked |
| IGMP Snooping | Fixed that when a port leave a group before , in the next time other ports leave specify group that port will affected |
| Event Log | When firmware upgrade error then event log will show invalid ip address. |
| IGMP Snooping | When set the IGMP snooping to enable then reboot DUT , the first IGMP join packet can't pass through to server port |
| Network DHCP | After restore with enable DHCP config file DUT will have two dhcp clients |
| NTP | When IPv4 can’t get ip and gateway address from DHCP server then NTP will not send IPv6 address packet |
| QinQ | Enable QinQ and set ether type to 0x8100, VLAN behavior will error, (0x8100)tag packet will be remove tag by DUT then drop |
| QoS | If set a port is ingress port and access mode(User Priority) will only marking P-Bit and send out but QoS not work |
| Port link / LED | Plug SFP-20TP/SFP-30TP WEB panel LED will blink show link-up even if not plug network line |
| Port link / LED | When change combo-port config from copper to fiber manual 1000M and plug SFP-20TP/SFP-30TP HW LED will always light even if not plug network line(about continue 2mins to 3mins) |
| Port link | If fiber port link-up in bootup then DUT will not bootup completely |
| RSTP | Fixed UI show RSTP statistics page RSTP received number place in STP received |
| Firmware Upgrade / FTP | UI use FTP to do firmware upgrade but only input user name without input password then upgrade will fail |
| Restore configure / Loop-detection | When enable loop-detection and detect port, and then backup confgure, the configure file loop-detection function was disable |

# Firmware Version 1.00.07

Date:  2015/11/20

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port link / LED | Port link-up to speed 100M then LED maybe can’t blink. |

# Firmware Version 1.00.06

Date:  2015/11/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| ACL configure | UI set ACL “IPv4 Source IP” and “IPv4 Destination IP” configuration will not verify input IP address is validity or not. |
| System / Port Statistics | Improve refresh port counter rate times. |
| Switch configure | Modified default config about” Statistics Polling Interval /Statistics Polling Ports” from 5 secs/5ports to 6 secs/12 ports. |
| IGMP Snooping | If IGMP Query Interval value is smaller than Query Response Interval due to bootup from old config or restore old config then convert query interval value to a new value. |
| IGMP Snooping | If IGMP Query Response Interval is over 255 due to bootup from old config or restore old config then convert query interval value to 255. |
| IGMP Snooping | UI can’t set Query interval smaller than Response interval but if Query interval is not bigger than Response interval(1/10) 5secs. then will not clear IGMP snooping table even not join packet. |
| IGMP Snooping | Fix the issue that igmp snooping receive igmpv3 packet with illegal value (number of group record) cause program crash |
| IGMP Snooping | Fix issue that group record number in igmpv3 packet be changed by igmp snooping module. |
| ACL configure | Set ACL configure of IPv4 Source IP or Destination IP must in index 1-128 but UI show wrong message. |
| IGMP Snooping | Let igmp snooping drop igmpv3 packets. |
| CLI / DHCP Snooping | CLI command “no ip sourceguard static-ip” can’t work. |

# Firmware Version 1.00.05

Date:  2015/10/27

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC Address | UI show MAC address in uppercase. |
| MAC Address Table | CLI show mac address table change place of “VID” and “MAC Address”. |
| IGMP Snooping | IGMP configuration “Query Response Interval” value range is 1-255 (1/10sec) |
| IGMP Snooping | IGMP configuration “Query interval” value should bigger than “Query Response Interval” |
| CLI / ACL configure | Change CLI command “acl no action” config from “deny” to “permit” and add command “acl action deny” |
| MAC Address Table | CLI add new commands “show mac address table all” |
| MAC Address Table | CLI add new commands “show mac address table top |
| Network configure | When change network mode from DHCP to Static mode, it will lose IP address (cannot access). |
| DHCP Snooping / Option 82 / Remote-ID | Fix the wrong packet content for remote id user manual string. |
| SSH | SSH function can’t not work. |
| DHCP Snooping / Option 82 / Circuit –ID | When default value with Option82_Circuit-ID, the packet of first byte should be display "00", but it display "01" |
| IGMP Snooping | IPTV will freeze and IGMP table suddenly empty. |
| DHCP Snooping / Option 82 / Remote-ID | WEB can’t show default DHCP OPT82 remote-id with MAC address. |
| Network DHCP | When change vender id DUT not send DHCP release packet. |
| IGMP Snooping | Multicast stream lost while client join/leave to same group lead to IPTV freezes. |
| SNMP / Event log | SNMP can not display the IPv4 address of the login user in the event log table correctly. |
| Restore configure / IGMP | After restore config igmp vlan id config would be set to default value even the igmp vlan id config is empty. |

# Firmware Version 1.00.04

Date:  2015/08/20

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| CLI / Port isolation | When CLI enable port-isolation then note will display “Port Isolation, Port Based vlan is automatically invalid.” |
| Restore configure / Port isolation | Restore config port-isolation can’t work successfully. |
| Port configuration | Fixed when combo port set to fiber then save config and reboot but that port will not blink when link-up. |

# Firmware Version 1.00.03

Date:  2015/08/19

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| WEB / Port isolation | UI modified port isolation config page/commands. |
| WEB / Port-based VLAN | UI display Port-based vlan group name from “Port Name” to “Name”. |

# Firmware Version 1.00.02

Date:  2015/08/12

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Loop-detection | Fixed issue that loop detect packet from other device under management VLAN would be received by module. |
| Loop-detection | Fixed issue that loop detect settings in module might be incorrect in some setting order. |
| Loop-detection | After a system loop happens, loop detection cannot successfully lock ports which the physical loop happen on. |
| Loop-detection | When loop-detection happened but LED blink frequency as normal link status. |

# Firmware Version 1.00.01

Date:  2015/08/10

| New Feature / System Release | Description |
| --- | --- |
| Loop Detection | Loop detection behavior change - “Block” port instead of “shut-down” port whenever the port is detected as looped-port. - Keep sending and “listening” loop-detection packet on the looped-port - Let the LED of looped-port blink at a cycle of 2-second period (1-second light, 1-second dark) - “Unblock” port whenever the system does not receive loop-detection packet on the looped-port for a period of “un-lock interval verification-period” |
| Update | Description |
| System / CLI session | CLI can support maximum 4 sessions in the same time. |
| System / SNMP | SNMP port status and port counter polling let it speed up. |
| Port-isolation | Port-isolation independence from port-based VLAN. |
| Loop Detection | Loop detection status if the port with trunk vlan id 7 then will display “Tagged-VLAN 7” otherwise it will only display “Untagged” when that port is access vlan. |
| Switch system | Telnet timeout can only set to 5-300 seconds but can’t set to disable now. |
| VLAN | In WEB/CLI/SNMP port based vlan includes CPU port option. |