| Model Name | FOS-3128 | Version | 1.00.11 | Designer | Approval |
| --- | --- | --- | --- | --- | --- |
| Part Name | Firmware | Date | 2017/04/14 |  |  |
| Part Number | N/A | Unit | N/A |  |  |
| File Name | FOS-3128_Release note_1.00.11_20170414.doc |  |  |  |  |

Release Note

Firmware Version 1.00.011	3

Firmware Version 1.00.10	3

Firmware Version 1.00.0Z	3

Firmware Version 1.00.0Y	3

Firmware Version 1.00.0X	4

Firmware Version 1.00.0W	4

Firmware Version 1.00.0V	4

Firmware Version 1.00.0U	5

Firmware Version 1.00.0T	5

Firmware Version 1.00.0S	6

Firmware Version 1.00.0R	6

Firmware Version 1.00.0Q	6

Firmware Version 1.00.0P	7

Firmware Version 1.00.0N	7

Firmware Version 1.00.0M	8

Firmware Version 1.00.0L	8

Firmware Version 1.00.0K	8

Firmware Version 1.00.0J	9

Firmware Version 1.00.0I	9

Firmware Version 1.00.0H	10

Firmware Version 1.00.0G	10

Firmware Version 1.00.0F	11

Firmware Version 1.00.0E	11

Firmware Version 1.00.0D	11

Firmware Version 1.00.0C	11

Firmware Version 1.00.0B	12

Firmware Version 1.00.0A	12

Firmware Version 1.00.09	12

Firmware Version 1.00.08	13

Firmware Version 1.00.07	13

Firmware Version 1.00.06	14

Firmware Version 1.00.05	14

Firmware Version 1.00.04	15

Firmware Version 1.00.03	15

Firmware Version 1.00.02	15

Firmware Version 1.00.01	16

# Firmware Version 1.00.011

Date:  2017/04/14

| New Feature / System Release | Description |
| --- | --- |
| System | Added that the time out can support the range from 1 to 1440 and the time (sec, minute) for console, SSH, Telnet. |
| Update | Description |
| N/A | N/A |

# Firmware Version 1.00.10

Date:  2017/04/12

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| SSH | Fixed that when enabled SSH then save and reboot, after boot up front console cannot input anything (SSH/WEB/SNMP working fine). |
| DHCPv6 | Fixed that when client port receives DHCPv6 confirm packet, it always will not forward to server trust port. |

# Firmware Version 1.00.0Z

Date:  2017/04/07

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| WEB | Modified that the time zone replaces GMT with UTC. |
| SSH | Fixed that Even log not show incorrect information when use SSH to login DUT. |
| SSH | Fixed that it can’t work fine issue. |

# Firmware Version 1.00.0Y

Date:  2017/03/31

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| SSH | Fixed that when using Putty or SecureCRT to access device via SSH in the first time will see "login as" then change to "Username" issue. |
| SSH | Modified that the user login count limitation to 3. |
| SSH | Fixed that it can’t work fine issue. |
| User log | Fixed that it can’t show IP address of user on ssh mode issue. |
| IP source binding | Fixed that when set IP source binding to enable then login device with ssh, the IP source binding can't work |

# Firmware Version 1.00.0X

Date:  2017/03/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| DHCP snooping | Fixed that it will crash when enabled DHCP snooping and server trust port receiving DHCPv6 packet with empty value. |

# Firmware Version 1.00.0W

Date:  2017/03/14

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| VLAN QinQ | Fixed that it can’t support double tag on packet when enable QinQ mode with ether type is 8100. |
| DHCP snooping | Fixed that it will crash when it receives a DHCP offer with a 9100 single vlan tag. |
| DHCP snooping | Fixed that it will crash when enabled DHCP snooping and server trust port receiving DHCPv6 packet with empty value. |

# Firmware Version 1.00.0V

Date:  2017/03/07

| New Feature / System Release | Description |
| --- | --- |
| Port Counter | Port counter allow to clear per port counter. |
| Update | Description |
| LLDP | Fixed that when enable lldp port config before lldp initial delay complete, should not cause first lldp packet sent out after 2 min ; LLDP packet will send out in 5 secs now. |
| User Authentication | Fixed that user can to see the other user’s password in “User Authentication” in html. |
| DHCP | Fixed that when receive double tag DHCP packet from Server Trust port(snooping enable/QinQ disable) then DHCP Snooping program will be die. |
| System | Fixed that after DUT boot up, the gateway will disappear. |

# Firmware Version 1.00.0U

Date:  2017/01/25

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Syslog message | Fixed that system log will loss when RSTP topology change. |

# Firmware Version 1.00.0T

Date:  2017/01/23

| New Feature / System Release | Description |
| --- | --- |
| System / Syslog message | Add system log manager to integrate all the syslog to avoid log loss when RSTP topology change |
| Update | Description |
| Syslog / Terminal History | Remove display IP address of syslog messages when use terminal history function in SSH mode. Before: <15>Dec 31 13:04:41 CLI: [admin][SSH][ip address]CLI command After: <15>Dec 31 13:04:41 CLI: [admin][SSH]CLI command: |
| Syslog / Terminal History | Fixed that when enable the terminal history function and access DUT from SSH. The syslog messages sent by the switch are incorrect. |
| User Authentication / RADIUS/TACACS Configuration | Fixed that cannot seek Tacacs+ Server if set IPv6 address in server2. |

# Firmware Version 1.00.0S

Date:  2017/01/06

| New Feature / System Release | Description |
| --- | --- |
| Syslog Coonfiguration | Release Syslog function - Terminal History |
| Syslog message | Support syslog log such as STP/LACP status change. |
| CPU & Memory | SNMP support to configure CPU loading. |
| Update | Description |
| User Authentication | Modify command from "Auth Method" to "Password Encryption". (All UI) |
| System / DHCP | Fixed that DHCP Program will not start if reboot DUT many times. |
| Static MAC Table | Fixed that when set Broadcast MAC address into Static MAC Table then will pop message with wrong word "Broadcast". |
| DHCP Snooping | Fixed that DHCP packet can't forwarding when enabled DHCP snooping. |
| DHCP Snooping | Fixed that board cannot get IP as DHCP client while DHCP snooping is enable with server trust port in trunk mode. |

# Firmware Version 1.00.0R

Date:  2016/09/07

| New Feature / System Release | Description |
| --- | --- |
| Security / PPPoE | PPPoE server trust ports |
| Update | Description |
| N/A | N/A |

# Firmware Version 1.00.0Q

Date:  2016/08/16

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| SSH | Fixed that SSH cannot work. |
| Syslog | Modify CLI show syslog also display “Mal attempt” words |

# Firmware Version 1.00.0P

Date:  2016/08/15

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| DHCP Snooping | Modify Security Configuration – “Filter configuration” - as this is DHCP snooping it should be called “DHCP snooping” |
| Syslog | Modify “Mal-attempt Log configuration” change name to “Syslog Configuration” |
| SNMP / MAC Table | Fixed SNMP get MAC table will display wrong value in first index and first time. |

# Firmware Version 1.00.0N

Date:  2016/07/29

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP Snooping | Modify WEB UI IGMP snooping title "IGMP/MLD snooping v3" --> "IGMPv3 / MLD v2 snooping". |
| MAC Address Table | Add CLI command : show mac address-table all <cr> |
| DHCP | Fixed that device will send 2 different DHCP session after restore configure |
| Syslog | Fixed that DUT can’t send syslog packet |
| SNMP / MIB | Fixed that SNMP IF-MIB (ifDescr / iName / ifAlias) display incorrect info |
| Port Status | Fixed that CLI support show port description but WEB UI "port status" page not show port description |
| IP Source Binding | Fixed that after enable IP source binding then we cannot access DUT from front console port |

# Firmware Version 1.00.0M

Date:  2016/07/14

| New Feature / System Release | Description |
| --- | --- |
| System / Firmware Upgrade | If firmware upgrade fail then check that image version number on UI after DUT bootup in another image, it should be null |
| Update | Description |
| RSTP | Modify RSTP default config - System Priority from 4096 to 32768. - Port Priority from 0 to 128. |
| System Information | Modify UI show switch information about firmware version title |

# Firmware Version 1.00.0L

Date:  2016/06/29

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| CLI / MAC Address Table | Fixed that CLI commands “show MAC address vlan” tip message show wrong word (speicfy) |
| MAC Address Table | Fixed that if traffic through port1 input then WEB show MAC address can’t jump to specify page |
| MAC Address Table | Fixed that WEB can’t clear specify port MAC address |

# Firmware Version 1.00.0K

Date:  2016/06/28

| New Feature / System Release | Description |
| --- | --- |
| System | If config partition damage in system bootup lead to DUT can’t access then will auto recovery config data . |
| CLI / terminal length | Add new commands for show show running-config、show default-config、show start-up-config can specify length terminal length [0-512] <cr> no terminal length <cr> show terminal <cr> |
| Update | Description |
| ACL | CLI add acl command “action permit” |
| DHCP Snooping / Option82 | CLI set option 82 formated commands is “ip dhcp snooping remote formatted” instead of “ip dhcp snooping formatted”. |
| CLI / MAC Table | Show MAC table with Sorting and counting functions Add: show mac address-table interface [port] <cr> show mac address-table mac [mac-addr] <cr> show mac address-table vlan [vlan_id] <cr> Remove: show mac address-table top <cr> show mac address-table all <cr> |
| ACL | Fixed that CLI can't set command "acl ingress-port any" successfully |
| ACL | Fixed that CLI remove "quit" command |
| Image info. | When do "show running-config" or "backup config", title of FW version always display image-1 version, even current is image-2. |

# Firmware Version 1.00.0J

Date:  2016/05/27

| New Feature / System Release | Description |
| --- | --- |
| System | Rewrite the process to avoid the issues of linux system or busybox |
| System | Auto-reboot system in case of out of memory suddenly |
| Update | Description |
| VLAN QinQ | CLI add commads about VLAN QinQ |
| LLDP | CLI modified show LLDP information title |
| RSTP Monitor | Fixed UI show RSTP statistics page RSTP received number place in STP received |
| FTP firmware upgrade | Fixed that UI use FTP to do firmware upgrade but only input user name without input password then upgrade will fail. |
| Backup Configure | Fixed that when enable loop-detection and detect port, and then backup confgure, the configure file loop-detection function was disable. |
| SNMP | Fixed can’t send power down trap package with IPv6 address |

# Firmware Version 1.00.0I

Date:  2016/03/03

| New Feature / System Release | Description |
| --- | --- |
| TACACS+ | Support user authentication “TACACS+” |
| System | Support user authentication method MD5 hashed password function |
| Update | Description |
| QoS | Fixed QoS can’t work if setting a port as ingress port with access mode will only marking P-Bit then send out. |
| Port Link | Fixed that if fiber port link-up when DUT boot up, it will fail. |
| LED | The LED on the WEB panel will blink show link-up when plugging the SFP-20TP/SFP-30TP but don’t plug the network line. |
| Combo port | When change combo-port from copper to fiber with manual 1000M and plug SFP LED will always light even if don’t plug network line(about continue 2mins to 3mins). |

# Firmware Version 1.00.0H

Date:  2016/02/02

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| DHCP Option 82 | Fixed that the OPT82 Formatted with wrong value under default value. |
| WEB UI | Fixed that when saving the configuration it appears “setting changed, still not saved”. |

# Firmware Version 1.00.0G

Date:  2016/02/01

| New Feature / System Release | Description |
| --- | --- |
| DHCP Option 82 | DHCP OPT 82 Formatted function and related CLI |
| Update | Description |
| MLD Snooping Status | Change all UI "V2 Done" to "Done" in MLD Snooping Status table |
| MAC limit | Fixed MAC limit issues with wrong procedure |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |
| Copper port | Fixed the wrong status when change the port status |
| QinQ Vlan Configuration | Fixed the wrong behavior with error when ether type is 0X8100 |
| Combo port | Fixed the wrong LED procedure when the port is linking |
| DHCP | Fixed that DUT will have multiple dhcp clients when restore and enable DHCP configuration file |
| CLI | Fixed that the command “acl action permit” can’t be entered. |

# Firmware Version 1.00.0F

Date:  2016/01/15

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC limit | Fixed MAC limit issues with wrong procedure |
| IGMP Snooping | Fixed that when enable IGMP snooping and reboot the IGMP join packet can't pass through to server port. |

# Firmware Version 1.00.0E

Date:  2016/01/11

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC limit | Fixed MAC limit issues with wrong procedure |

# Firmware Version 1.00.0D

Date:  2016/01/05

| New Feature / System Release | Description |
| --- | --- |
| MAC limit | MAC limit per port in WEB and CLI |
| Update | Description |
| Power Down trap | Modify that the power down trap send traps from 3 times/1min to 3 times/ 1 sec |
| Firmware update | Fixed that when firmware upgrade error then event log will show invalid ip address |

# Firmware Version 1.00.0C

Date:  2015/12/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Switch Configuration | Modified the default configuration about ” Statistics Polling Interval /Statistics Polling Ports” from 5 secs/5ports to 6 secs/12 ports |
| RSTP | Fixed that RSTP port can’t be blocked when related port enable again and sent 1 pps packet from other port |

# Firmware Version 1.00.0B

Date:  2015/12/16

| New Feature / System Release | Description |
| --- | --- |
| WEB UI | Enable or disable IGMPv3 function. |
| Update | Description |
| ACL | Modify the command in CLI of “action” to accept “mirror or copy or copy(mirror)” |
| CLI | Fixed the command “no ip sourceguard static-ip” can’t work |
| MAC address table | Fixed that when all UI set same MAC address and same VID at different port in Static MAC address table, system not pop warning message even setting not success |
| MAC address table | Fixed that CLI/WEB/SNMP can’t set broadcast MAC address in Static MAC address table. |
| IP address Table | Fixed that the entry in the Static IP Table Configuration can’t be deleted by the telnet command. |
| SNMP | Fixed that SNMP can’t add IPv4 address into DHCP Snooping table. |
| 802.1X | Fixed that The 802.1X statistics table can’t be updated. |

# Firmware Version 1.00.0A

Date:  2015/10/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| ACL | Make user configure ACL IPv4 source IP or Destination IP must in index |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |

# Firmware Version 1.00.09

Date:  2015/10/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |

# Firmware Version 1.00.08

Date:  2015/10/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP snooping | Modify IGMP configuration “query response interval” value rage from 1-6000 (1/10sec) to 1-255 (1/10sec) |
| IGMP snooping | Change IGMP “query interval” default setting to be 125 sec |
| IGMP snooping | Change IGMP “query response interval” to 100 (1/10sec). |
| IGMP snooping | Fixed when configure IGMP query interval parameters over 256 sec , it will cause the parameters reset from “0” Ex: 256 seconds becomes 0, 257 becomes 1 etc….. |
| IGMP snooping | Fixed in WEB UI shows wrong query “response” interval string |

# Firmware Version 1.00.07

Date:  2015/10/26

| New Feature / System Release | Description |
| --- | --- |
| MAC address table | CLI add new commands “show mac address table all” and “show mac address table top” |
| Update | Description |
| MAC address table | UI show MAC address in uppercase |
| MAC address table | Change CLI show MAC address table “VID” and “MAC” field location. (exchange VID and MAC filed location) |
| IGMP snooping | Add IGMP configuration “query interval” parameters should bigger than “query response interval” parameters rule. |
| Network configuration | When change device IP address form DHCP to static IP, device cannot use the static IP to access. |
| Even t log | Fixed in CLI UI event log will cut long message character issue. |
| Traffic forwarding | Fixed When enable IGMP snooping and DHCP snooping it will cause port 33 cannot forward DHCP offer packets and IGMP control packets. |
| CLI | Improve CLI “reload” command. |
| SFP port | Fixed some specified SFP transceiver cannot link up issue. |
| Port isolation | Add CPU can be include when configure Port Based VLAN. |
| Port isolation | port-based vlan group name on UI from “Port Name” change to “Name”. |
| Port isolation | WEB UI port isolation page from “VLAN configuration” move to “Security Configuration” page. |
| Port isolation | CLI add new command “security port-isolation” “security port-isolation up-link-port” “show security port-isolation” |
| Port isolation | Fixed Restore config port-isolation can not work successfully |
| Loop detection | Fixed When loop happened the event log not show the complete event log message. eg.. some characters may be cut off. |
| IPv6 | Fixed When use IPv6 with FTP protocol to upgrade firmware or auto-provision , it will unsuccessfully. |
| Loop detection | Fixed loop packet from other device under management VLAN would not block. |
| Loop detection | When port looped the loop detection cannot successfully lock port when it connected to another FOS-3128 loop detection enable port. |
| Combo port interface | Fixed when combo port set to fiber then save config and reboot. It will cause port LED not blink when link up. |
| SSH | Fixed SSH function cannot work |
| DHCP option 82 | Fixed DHCP option 82 when manual configure remote and circuit ID it will have wrong content format. |

# Firmware Version 1.00.06

Date:  2015/08/03

| New Feature / System Release | Description |
| --- | --- |
| Loop detection | Behavior change : 1.Looped port from “port shutdown” change to listening 2.Looped port LED will blink at a cycle of 2-second period (Orange light flash) 3.Unblock looped port situation under the looped port not receive loop detection packet. |
| Update | Description |
| Telnet | CLI can support maximum 4 sessions at the same time |
| SNMP | Increase device response SNMP port status and counter speed. |
| Port isolation | Separate Port based VLAN and VLAN isolation config |

# Firmware Version 1.00.05

Date:  2015/07/09

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP/MLD snooping | Fixed IGMP/MLD VLAN ID cannot enable “snooping” and “Querying” function |

# Firmware Version 1.00.04

Date:  2015/07/08

| New Feature / System Release | Description |
| --- | --- |
| System | New add show CPU utilization and memory usage rate function. CLI : Switch# show switch-info cpu-mem-statistice |
| Update | Description |
| DHCP option 82 | Fixed CLI cannot enter 63 characters in DHCP option 82 remote ID and circuit ID |

# Firmware Version 1.00.03

Date:  2015/06/15

| New Feature / System Release | Description |
| --- | --- |
| Counter polling | Can manual set port counter polling interval time and ports |
| DHCP Option82 | Support manual edit DHCP option 82 circuit and remote ID. |
| WEB UI | Support firmware upgrade and config backup /restore via HTTP |
| Update | Description |
| SNMP | Power down trap source IP are wrong |

# Firmware Version 1.00.02

Date:  2015/05/05

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC address | Allow MAC address table aging time can up to 172800 secs (2days) |
| System | Device sent DHCP discover packet before success boot up. |
| MAC | Fixed by port show MAC address table will dump error information . |

# Firmware Version 1.00.01

Date:  2015/04/01

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Auto-provision | Fixed after load factory default , Auto Provision cannot restore config file. |
| VLAN | Fixed VLAN name of VID 10 cannot shown in “IEEE 802.1q Tag VLAN table” |
| RSTP | When enable RSTP aggregated port function on a vlan trunking port , it will cause the traffic cannot pass-through the port. |