| Model Name | FOS-3148 | Version | 1.00.0V | Designer | Approval |
| --- | --- | --- | --- | --- | --- |
| Part Name | Firmware | Date | 2015/11/19 |  |  |
| Part Number | N/A | Unit | N/A |  |  |
| File Name | FOS-3148_Release note_1.00.0V_20151119.doc |  |  |  |  |

Release Note

Firmware Version 1.00.0V	3

Firmware Version 1.00.0U	3

Firmware Version 1.00.0T	3

Firmware Version 1.00.0S	3

Firmware Version 1.00.0Q	4

Firmware Version 1.00.0P	4

Firmware Version 1.00.0N	5

Firmware Version 1.00.0M	5

Firmware Version 1.00.0L	6

Firmware Version 1.00.0K	6

Firmware Version 1.00.0J	6

Firmware Version 1.00.0I	7

Firmware Version 1.00.0H	7

Firmware Version 1.00.0G	7

Firmware Version 1.00.0F	8

Firmware Version 1.00.0E	8

Firmware Version 1.00.0D	8

Firmware Version 1.00.0C	9

Firmware Version 1.00.0B	9

Firmware Version 1.00.0A	9

Firmware Version 1.00.09	10

Firmware Version 1.00.08	10

Firmware Version 1.00.07	10

Firmware Version 1.00.06	11

Firmware Version 1.00.05	11

Firmware Version 1.00.04	12

Firmware Version 1.00.03	12

Firmware Version 1.00.02	12

Firmware Version 1.00.01	13

# Firmware Version 1.00.0V

Date:  2015/11/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port Interface | Fixed that Port link-up to speed 100M then LED maybe can’t blink |

# Firmware Version 1.00.0U

Date:  2015/11/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port Counter | Improve refresh port counter rate times |
| Switch Configuration | Modified the default configuration about ” Statistics Polling Interval /Statistics Polling Ports” from 5 secs/5ports to 6 secs/12 ports |
| CLI | Fixed the command “no ip sourceguard static-ip” can’t work |

# Firmware Version 1.00.0T

Date:  2015/10/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| ACL | Make user configure ACL IPv4 source IP or Destination IP must in index |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |

# Firmware Version 1.00.0S

Date:  2015/10/29

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| ACL | Fixed when config ACL “IPv4 source IP” and “IPv4 Destination IP” UI are not verify input IP address is validity or not. |
| IGPM Snooping | Fixed after restore config ,IGMP vlan setting will become default |
| IGMP Snooping | When IGMP Query interval is less than Query Response interval , after reboot or restore config it will make IGMP query interval value change. |
| IGMP Snooping | Modify IGMP query response interval parameter config range: From 1-6000 change to 1-255 |
| IGMP Snooping | Fixed IGMP snooping table not clear under Query interval less than response interval. |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |

# Firmware Version 1.00.0Q

Date:  2015/10/23

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP snooping | Fixed IGMP snooping table suddenly empty issue. Fixed when receive the same multicast channel join/leave from client port at the same time , it will cause the watch the same multicast channel’s STB picture freezes. |

# Firmware Version 1.00.0P

Date:  2015/10/15

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| DHCP option 82 | Fixed DHCP option 82 circuit-ID first byte value from “01” modify to “01” |
| IGMP snooping | Fixed IGMP snooping table suddenly empty issue. |
| DHCP option 82 | Fixed WEB UI cannot show default DHCP option 82 remote-ID with MAC address issue. |

# Firmware Version 1.00.0N

Date:  2015/10/05

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP snooping | Modify IGMP configuration “query response interval” value rage from 1-6000 (1/10sec) to 1-255 (1/10sec) |
| IGMP snooping | Add IGMP configuration “query interval” parameters should bigger than “query response interval” parameters rule. |
| IGMP snooping | Change IGMP “query interval” default setting to be 125 sec |
| IGMP snooping | Change IGMP “query response interval” to 100 (1/10sec). |
| IGMP snooping | Fixed when configure IGMP query interval parameters over 256 sec , it will cause the parameters reset from “0” Ex: 256 seconds becomes 0, 257 becomes 1 etc….. |
| IGMP snooping | Fixed in WEB UI shows wrong query “response” interval string |
| STP,RSTP,LLDP,LACP | Fixed STP RSTP , LLDP , LACP cannot work on port-33-44. |
| DHCP option 82 | Fixed DHCP option 82 when manual configure remote and circuit ID it will have wrong content format. |
| SSH | Fixed SSH function cannot work |

# Firmware Version 1.00.0M

Date:  2015/10/05

| New Feature / System Release | Description |
| --- | --- |
| MAC address table | CLI add new commands “show mac address table all” and “show mac address table top” |
| Update | Description |
| MAC address table | UI show MAC address in uppercase |
| MAC address table | Change CLI show MAC address table “VID” and “MAC” field location. (exchange VID and MAC filed location) |
| Combo port interface (port 45-48) | Fixed when combo port set to fiber then save config and reboot. It will cause port LED not blink when link up. |
| Network configuration | When change device IP address form DHCP to static IP, device cannot use the static IP to access. |
| Even t log | Fixed in CLI UI event log will cut long message character issue. |
| Traffic forwarding | Fixed When enable IGMP snooping and DHCP snooping it will cause port 33 cannot forward DHCP offer packets and IGMP control packets. |

# Firmware Version 1.00.0L

Date:  2015/8/21

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | Fixed Restore config port-isolation can not work successfully |
| CLI | Improve CLI “reload” command. |
| SFP port | Fixed some specified SFP transceiver cannot link up issue. |

# Firmware Version 1.00.0K

Date:  2015/08/18

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | CLI cannot set port isolation config. |

# Firmware Version 1.00.0J

Date:  2015/08/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | port-based vlan group name on UI from “Port Name” change to “Name”. |
| Port isolation | WEB UI port isolation page from “VLAN configuration” move to “Security Configuration” page. |
| Port isolation | CLI add new command “security port-isolation” “security port-isolation up-link-port” “show security port-isolation” |
| Loop detection | Fixed When loop happened the event log not show the complete event log message. eg.. some characters may be cut off. |
| IPv6 | Fixed When use IPv6 with FTP protocol to upgrade firmware or auto-provision , it will unsuccessfully. |
| Loop detection | Fixed loop packet from other device under management VLAN would not block. |
| Loop detection | When port looped the loop detection cannot successfully lock port when it connected to another FOS-3148 loop detection enable port. |

# Firmware Version 1.00.0I

Date:  2015/08/3

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | Improve Port based vlan must at least one index includes CPU port. |

# Firmware Version 1.00.0H

Date:  2015/07/31

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | Add CPU can be include when configure Port Based VLAN. |

# Firmware Version 1.00.0G

Date:  2015/07/24

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| SNMP | Fixed SNMP trap, event log and syslog display loop detection status words length over permissible range |

# Firmware Version 1.00.0F

Date:  2015/07/24

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| VLAN | Fixed when Management VLAN port only trunk mode , after output config file it will lose Management port setting. |

# Firmware Version 1.00.0E

Date:  2015/07/22

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Telnet | Fixed Telnet timeout cannot set to disable. |
| Config restore | When device restore a config file with incorrect VLAN command content. It will cause device restore process fail. |
| SNMP | Fixed when receive get-next-request OID is 1.3.6.1.2.1.17.4.2.0(dot1dTp aging time) will cause SNMP service disable.- |

# Firmware Version 1.00.0D

Date:  2015/07/17

| New Feature / System Release | Description |
| --- | --- |
| System | New add show CPU utilization and memory usage rate function. CLI : Switch# show switch-info cpu-mem-statistice |
| Update | Description |
| Loop detection | Behavior change : 1.Looped port from “port shutdown” change to listening 2.Looped port LED will blink at a cycle of 2-second period (Orange light flash) 3.Unblock looped port situation under the looped port not receive loop detection packet. |
| Telnet | CLI can support maximum 4 sessions at the same time |
| SNMP | Increase device response SNMP port status and counter speed. |
| Port isolation | Separate Port based VLAN and VLAN isolation config |
| DHCP option 82 | Fixed CLI cannot enter 63 characters in DHCP option 82 remote ID and circuit ID |
| IGMP/MLD snooping | Fixed IGMP/MLD VLAN ID cannot enable “snooping” and “Querying” function |

# Firmware Version 1.00.0C

Date:  2015/06/05

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | Fixed after restore config then reboot , device will change to default setting |
| SFP port | When plug-in 100M UTP (RJ-45) type SFP, the link LED shows link-up (light) without connected UTP cable. |
| SNMP | Power down trap source IP are wrong |

# Firmware Version 1.00.0B

Date:  2015/05/26

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | When save config will lead to CLI can not access. |

# Firmware Version 1.00.0A

Date:  2015/05/04

| New Feature / System Release | Description |
| --- | --- |
| Firmware upgrade | WEB UI support HTTP firmware upgrade and config restore. |
| Update | Description |
| SNMP | Speed up show MIB counter. |
| SNMP | Fixed device not sent “cold-start” trap. |

# Firmware Version 1.00.09

Date:  2015/05/04

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC address table | Improve MAC address aging time can config up to 172800 seconds |
| System information | Improve FAN status display on show system information. |
| DHCP | Fixed device boot up and send DHCP DISCOVER packet before port link up. |
| System | Fixed when SFP slot insert Copper port SFP without link up , port of LED will show green light. |
| MAC address | Fixed show MAC address table by port will cause error. |
| SNMP | Fixed SNMP will send battery “Charge “ , “Discharge” trap twice. |

# Firmware Version 1.00.08

Date:  2015/3/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| DHCP auto-provision | Fixed after device load to default factory setting , DHCP auto-provision cannot success update config file |

# Firmware Version 1.00.07

Date:  2015/3/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| SNTP | Fixed show SNTP status typo “STNP” |
| SNMP | Fixed SNMP statistics polling interval time cannot set to 60 seconds. |

# Firmware Version 1.00.06

Date:  2015/3/23

| New Feature / System Release | Description |
| --- | --- |
| DHCP Snooping | Add DHCP option82 circuit and remote ID value can be manual configuration. |
| Update | Description |
| WEB UI | Fixed MAC address table no shows update information when press “Next” or “Top” button twice. |
| WEB UI | Fixed Port 25-28 when do port shutdown still shows link up status |
| SNMP | After device restore config and reboot , the SNMP can not modify any setting. |
| Default gateway | Allow default gateway IP can configurable to 0.0.0.0 IP address. |
| SNMP | Fixed SNMP allow config port under 1000M speed can working on half duplex mode. |
| Loop detection | Fixed loop detection did not correct block loop port. |
| LLDP | Fixed LLD system-name will lose config setting after device reboot. |
| LLDP | Fixed When config LLDP system-name over 55 characters it will overwrite LLDP system-description config characters. |
| Q-in-Q | Fixed Q-in-Q ISP port will not add S-tag under following situation. a.Q-in-Q client port enable DHCP snooping receive DHCP packet. b. IGMP control packet outgoing from Q-in-Q ISP port snooping enable when outgoing from Q-in-Q ISP port IGMP control packet |
| IGMP snooping | Fixed IGMP group table show empty value, when it receive IGMP join packet that out of 224~239.x.x.x multicast IP rage. |
| IGMP snooping | Fixed IGMP group table cannot remove 227.1.1.10 multicast group when it does not receive any IGMP join anymore. |
| Port counter | Modified port counter oversize packet as ‘over Max byte’. |

# Firmware Version 1.00.05

Date:  2015/3/12

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | Fixed when show 802.1x port status via SNMP , it will cause device slowly response SNMP polling and CLI malfunction. |
| WEB UI | Fixed ACL configuration page in WEB UI without delete button. |
| WEB UI | Improve ACL configuration page in WEB UI display. |
| IPv6 | Solicit and Request not included DHCP vendor ID in DHCPv6 option on default value , but after change ID will include. |
| ACL | Improve ACL rule port deny ingress and permit behavior. |
| LLDP | Fixed LLDP cannot shows IPv6 address. |

# Firmware Version 1.00.04

Date:  2015/3/07

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Layer 2 BPDU packet filter | Fixed when enable layer 2 BPDU packets filter cannot filter LLDP packets. |
| Q-in-Q | Allow Q-in-Q ISP port can pass through Single Tag VLAN traffic. |
| IGMP snooping | Fixed When enable multiple VLAN on IGMP groups , it will cause snooping client cannot received IGMP query. |

# Firmware Version 1.00.03

Date:  2015/3/07

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| WEB UI | Improve WEB UI Switch panel renew frequency. |

# Firmware Version 1.00.02

Date:  2015/3/05

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | Fixed when IGMP snooping features enable for a period of time , device will automatically reboot. |
| IP source guard | Fixed when IP source guard enable and config to dhcp mode, it will cause the clients cannot get IPv6 address from DHCP server. |

# Firmware Version 1.00.01

Date:  2015/1/23

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| N/A | N/A |
| RSTP | Reduce system resource usage when enable RSTP feature. |
| Packet forward | Fixed DHCPv6 cannot send SOLICIT packet. |
| IPv6 | Fixed IPv6 global address will invalid when save config and reboot. |