| Model Name | FOS-3148 | Version | 1.00.14 | Designer | Approval |
| --- | --- | --- | --- | --- | --- |
| Part Name | Firmware | Date | 2017/04/17 |  |  |
| Part Number | N/A | Unit | N/A |  |  |
| File Name | FOS-3148_Release note_1.00.14_20170417.doc |  |  |  |  |

Release Note

Firmware Version 1.00.14	3

Firmware Version 1.00.13	3

Firmware Version 1.00.12	3

Firmware Version 1.00.11	4

Firmware Version 1.00.10	6

Firmware Version 1.00.0Z	6

Firmware Version 1.00.0Y	7

Firmware Version 1.00.0X	7

Firmware Version 1.00.0W	8

Firmware Version 1.00.0V	8

Firmware Version 1.00.0U	8

Firmware Version 1.00.0T	9

Firmware Version 1.00.0S	9

Firmware Version 1.00.0Q	10

Firmware Version 1.00.0P	10

Firmware Version 1.00.0N	10

Firmware Version 1.00.0M	11

Firmware Version 1.00.0L	11

Firmware Version 1.00.0K	12

Firmware Version 1.00.0J	12

Firmware Version 1.00.0I	12

Firmware Version 1.00.0H	13

Firmware Version 1.00.0G	13

Firmware Version 1.00.0F	13

Firmware Version 1.00.0E	13

Firmware Version 1.00.0D	14

Firmware Version 1.00.0C	14

Firmware Version 1.00.0B	15

Firmware Version 1.00.0A	15

Firmware Version 1.00.09	15

Firmware Version 1.00.08	16

Firmware Version 1.00.07	16

Firmware Version 1.00.06	16

Firmware Version 1.00.05	17

Firmware Version 1.00.04	17

Firmware Version 1.00.03	18

Firmware Version 1.00.02	18

Firmware Version 1.00.01	18

# Firmware Version 1.00.14

Date:  2017/04/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| WEB | Fixed that WEB can’t configure CLI and WEB timeout issue. |

# Firmware Version 1.00.13

Date:  2017/04/13

| New Feature / System Release | Description |
| --- | --- |
| System | Added that the time out can support the range from 1 to 1440 and the time (sec, minute) for console, SSH, Telnet. |
| Update | Description |
| WEB | Modified that the time zone replaces GMT with UTC. |
| User log | Fixed that it can’t show IP address of user on ssh mode issue. |
| VLAN QinQ | Fixed that it can’t support double tag on packet when enable QinQ mode with ether type is 8100. |
| DHCP snooping | Fixed that it will crash when it receives a DHCP offer with a 9100 single vlan tag. |
| DHCP snooping | Fixed that it will crash when enabled DHCP snooping and server trust port receiving DHCPv6 packet with empty value. |
| SSH | Fixed that when using Putty or SecureCRT to access device via SSH in the first time will see "login as" then change to "Username" issue. |
| SSH | Modified that the user login count limitation to 3. |
| SSH | Fixed that it can’t work fine issue. |
| DHCPv6 | Fixed that when client port receives DHCPv6 confirm packet, it always will not forward to server trust port. |

# Firmware Version 1.00.12

Date:  2017/03/01

| New Feature / System Release | Description |
| --- | --- |
| System Service | SNMP trap & syslog can support CPU load. |
| Syslog | Support more detail only on syslog such as STP/LACP status change. |
| Port Counter | Port counter allow to clear per port counter. |
| Update | Description |
| User Authentication | Modify all UI about MD5 hashed password title/command/desc. |
| WEB | Modify to remove the display of ip address in user log function within SSH mode |
| HTTP firmware upgrade | Fixed that use HTTP to do firmware upgrade and backup/restore config fail. |
| Load Factory Settings | Fixed that when execute “Load Factory Settings Except Network Configuration” will resend dhcpv6 Solicit packet. |
| SNMP | Fixed that if port is link down but use SNMP to show port status also have speed value, after modified it is display zero now. |
| Syslog | Fixed if enable the terminal history function and access DUT from SSH. The syslog messages sent by the switch are incorrect. |
| LLDP | Fixed that when enable lldp port config before lldp initial delay complete, should not cause first lldp packet sent out after 2 min ; LLDP packet will send out in 5 secs now. |
| User Authentication | Fixed that user can to see the other user’s password in “User Authentication” in html. |
| DHCP | Fixed that when receive double tag DHCP packet from Server Trust port(snooping enable/QinQ disable) then DHCP Snooping program will be die. |

# Firmware Version 1.00.11

Date:  2016/12/07

| New Feature / System Release | Description |
| --- | --- |
| Terminal Length | CLI add new commands for show show running-config、show default-config、show start-up-config can specify length: terminal length [0-512] <cr> no terminal length <cr> show terminal <cr> |
| MAC Address Table | Show MAC table with sorting and counting functions: Add: show mac address-table interface [port] <cr> show mac address-table mac [mac-addr] <cr> show mac address-table vlan [vlan_id] <cr> Remove: show mac address-table top <cr> |
| System check | If firmware upgrade fail ( kernel partition damage) then image version number on UI will display empty. |
| PPPoE server trust ports | Add new feature / PPPoE server trust ports |
| Syslog Configuration | Add new feature / User log function. (Enable/Disable record user keying CLI commands) CLI commands: syslog logging-type terminal-history. no syslog logging-type terminal-history. |
| Update | Description |
| System Service | WEB UI / SSH service take out of drop down menu. |
| Security Configuration | Rename “Filter configuration” to “DHCP Snooping”. |
| Syslog Configuration | Rename “Mal-attempt Log configuration” to “Syslog Configuration”. |
| IGMP Snooping | Rename “IGMP/MLD snooping v3” to “IGMPv3/MLDv2 snooping”. |
| RSTP | Modify RSTP default config: System Priority from 4096 to 32768. Port Priority from 0 to 128. |
| DHCP Snooping / Option 82 | Modify CLI set option 82 formated commands Is “ip dhcp snooping remote formatted” instead of “ip dhcp snooping formatted”. |
| SNMP | Modify SNMP if-MIB (ifDescr / iName / ifAlias) display incorrect info. |
| Switch Port Status | Add show port description info. |
| IP Source Binding | Fixed that after enable IP source binding then cannot access DUT from front console port. |
| Configure file | Fixed that when do "show running-config" or "backup config", title of FW version always display image-1 version, even current is image-2. |
| DHCP | Fixed that Device will send 2 different DHCP session after restore configure(manual/auto-provision). |
| Syslog | Fixed that DUT can’t send syslog packet. |
| DHCP Snooping | Fixed that DHCP packet can't forwarding when enabled DHCP snooping. |

# Firmware Version 1.00.10

Date:  2016/06/03

| New Feature / System Release | Description |
| --- | --- |
| TACACS+ | Support user authentication “TACACS+” |
| User authentication method | Support user authentication method MD5 hashed password function |
| System | Rewrite the process to avoid the issues of linux system or busybox |
| System | Auto-reboot system in case of out of memory suddenly |
| Update | Description |
| VLAN QinQ | CLI add commads about VLAN QinQ |
| LLDP | CLI modified show LLDP information title |
| ACL | CLI add command “action permit” |
| QoS | Fixed that access mode(User Priority) will only marking P-Bit and send out but QoS not work. |
| LED | Fixed that when Plug SFP-20TP/SFP-30TP WEB panel LED will blink show link-up even if not plug network line. |
| LED | Fixed that when change combo-port config from copper to fiber manual 1000M and plug SFP-20TP/SFP-30TP HW LED will always light even if not plug network line |
| Port link | Fixed that if fiber port link-up in bootup then DUT will not bootup completely |
| RSTP | Fixed UI show RSTP statistics page RSTP received number place in STP received |
| FTP firmware upgrade | Fixed that UI use FTP to do firmware upgrade but only input user name without input password then upgrade will fail. |
| Backup Configure | Fixed that when enable loop-detection and detect port, and then backup confgure, the configure file loop-detection function was disable. |
| CLI | Fixed that CLI can't set command "acl ingress-port any" successfully |
| CLI | CLI remove "quit" command. |

# Firmware Version 1.00.0Z

Date:  2016/02/02

| New Feature / System Release | Description |
| --- | --- |
| DHCP Option 82 | DHCP OPT 82 Formatted function and related CLI |
| Update | Description |
| MLD Snooping Status | Change all UI "V2 Done" to "Done" in MLD Snooping Status table |
| MAC limit | Fixed MAC limit issues with wrong procedure |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |
| Copper port | Fixed the wrong status when change the port status |
| QinQ Vlan Configuration | Fixed the wrong behavior with error when ether type is 0X8100 |
| IGMP Snooping | Fixed that when enable IGMP snooping and reboot the IGMP join packet can't pass through to server port. |
| DHCP | Fixed that DUT will have multiple dhcp clients when restore and enable DHCP configuration file |
| CLI | Fixed that the command “acl action permit” can’t be entered |

# Firmware Version 1.00.0Y

Date:  2016/01/13

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC limit | Fixed MAC limit issues with wrong procedure |

# Firmware Version 1.00.0X

Date:  2016/01/11

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC limit | Fixed MAC limit issues with wrong procedure |
| DHCP Option 82 | Fixed that it will fail when enable too much option 82 circuit port at the same time on web |
| Event Log | Fixed that when firmware upgrade error then event log will show invalid ip address |

# Firmware Version 1.00.0W

Date:  2016/01/05

| New Feature / System Release | Description |
| --- | --- |
| MAC limit | MAC limit per port in WEB and CLI |
| IGMPv3 | Enable or Disable IGMPv3 function |
| Update | Description |
| Power Down trap | Modify that the power down trap send traps from 3 times/1min to 3 times/ 1 sec |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |
| ACL | Modify the related command about ACL function in CLI |
| MAC address table | Fixed that when all UI set same MAC address and same VID at different port in Static MAC address table, system not pop warning message even setting not success |
| MAC address table | Fixed that CLI/WEB/SNMP can’t set broadcast MAC address in Static MAC address table. |
| IP address Table | Fixed that the entry in the Static IP Table Configuration can’t be deleted by the telnet command |
| SNMP | Fixed that SNMP can’t add IPv4 address into DHCP Snooping table. |
| 802.1X | Fixed that The 802.1X statistics table can’t be updated. |
| RSTP | Fixed that RSTP port can’t be blocked when related port enable again and sent 1 pps packet from other port |

# Firmware Version 1.00.0V

Date:  2015/11/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port Interface | Fixed that Port link-up to speed 100M then LED maybe can’t blink |

# Firmware Version 1.00.0U

Date:  2015/11/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port Counter | Improve refresh port counter rate times |
| Switch Configuration | Modified the default configuration about ” Statistics Polling Interval /Statistics Polling Ports” from 5 secs/5ports to 6 secs/12 ports |
| CLI | Fixed the command “no ip sourceguard static-ip” can’t work |

# Firmware Version 1.00.0T

Date:  2015/10/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| ACL | Make user configure ACL IPv4 source IP or Destination IP must in index |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |

# Firmware Version 1.00.0S

Date:  2015/10/29

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| ACL | Fixed when config ACL “IPv4 source IP” and “IPv4 Destination IP” UI are not verify input IP address is validity or not. |
| IGPM Snooping | Fixed after restore config ,IGMP vlan setting will become default |
| IGMP Snooping | When IGMP Query interval is less than Query Response interval , after reboot or restore config it will make IGMP query interval value change. |
| IGMP Snooping | Modify IGMP query response interval parameter config range: From 1-6000 change to 1-255 |
| IGMP Snooping | Fixed IGMP snooping table not clear under Query interval less than response interval. |
| IGMP Snooping | Fixed IGMP snooping malfunction when receive IGMPv3 packet with illegal value (number of group record) |

# Firmware Version 1.00.0Q

Date:  2015/10/23

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP snooping | Fixed IGMP snooping table suddenly empty issue. Fixed when receive the same multicast channel join/leave from client port at the same time , it will cause the watch the same multicast channel’s STB picture freezes. |

# Firmware Version 1.00.0P

Date:  2015/10/15

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| DHCP option 82 | Fixed DHCP option 82 circuit-ID first byte value from “01” modify to “01” |
| IGMP snooping | Fixed IGMP snooping table suddenly empty issue. |
| DHCP option 82 | Fixed WEB UI cannot show default DHCP option 82 remote-ID with MAC address issue. |

# Firmware Version 1.00.0N

Date:  2015/10/05

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| IGMP snooping | Modify IGMP configuration “query response interval” value rage from 1-6000 (1/10sec) to 1-255 (1/10sec) |
| IGMP snooping | Add IGMP configuration “query interval” parameters should bigger than “query response interval” parameters rule. |
| IGMP snooping | Change IGMP “query interval” default setting to be 125 sec |
| IGMP snooping | Change IGMP “query response interval” to 100 (1/10sec). |
| IGMP snooping | Fixed when configure IGMP query interval parameters over 256 sec , it will cause the parameters reset from “0” Ex: 256 seconds becomes 0, 257 becomes 1 etc….. |
| IGMP snooping | Fixed in WEB UI shows wrong query “response” interval string |
| STP,RSTP,LLDP,LACP | Fixed STP RSTP , LLDP , LACP cannot work on port-33-44. |
| DHCP option 82 | Fixed DHCP option 82 when manual configure remote and circuit ID it will have wrong content format. |
| SSH | Fixed SSH function cannot work |

# Firmware Version 1.00.0M

Date:  2015/10/05

| New Feature / System Release | Description |
| --- | --- |
| MAC address table | CLI add new commands “show mac address table all” and “show mac address table top” |
| Update | Description |
| MAC address table | UI show MAC address in uppercase |
| MAC address table | Change CLI show MAC address table “VID” and “MAC” field location. (exchange VID and MAC filed location) |
| Combo port interface (port 45-48) | Fixed when combo port set to fiber then save config and reboot. It will cause port LED not blink when link up. |
| Network configuration | When change device IP address form DHCP to static IP, device cannot use the static IP to access. |
| Even t log | Fixed in CLI UI event log will cut long message character issue. |
| Traffic forwarding | Fixed When enable IGMP snooping and DHCP snooping it will cause port 33 cannot forward DHCP offer packets and IGMP control packets. |

# Firmware Version 1.00.0L

Date:  2015/8/21

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | Fixed Restore config port-isolation can not work successfully |
| CLI | Improve CLI “reload” command. |
| SFP port | Fixed some specified SFP transceiver cannot link up issue. |

# Firmware Version 1.00.0K

Date:  2015/08/18

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | CLI cannot set port isolation config. |

# Firmware Version 1.00.0J

Date:  2015/08/17

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | port-based vlan group name on UI from “Port Name” change to “Name”. |
| Port isolation | WEB UI port isolation page from “VLAN configuration” move to “Security Configuration” page. |
| Port isolation | CLI add new command “security port-isolation” “security port-isolation up-link-port” “show security port-isolation” |
| Loop detection | Fixed When loop happened the event log not show the complete event log message. eg.. some characters may be cut off. |
| IPv6 | Fixed When use IPv6 with FTP protocol to upgrade firmware or auto-provision , it will unsuccessfully. |
| Loop detection | Fixed loop packet from other device under management VLAN would not block. |
| Loop detection | When port looped the loop detection cannot successfully lock port when it connected to another FOS-3148 loop detection enable port. |

# Firmware Version 1.00.0I

Date:  2015/08/3

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | Improve Port based vlan must at least one index includes CPU port. |

# Firmware Version 1.00.0H

Date:  2015/07/31

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Port isolation | Add CPU can be include when configure Port Based VLAN. |

# Firmware Version 1.00.0G

Date:  2015/07/24

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| SNMP | Fixed SNMP trap, event log and syslog display loop detection status words length over permissible range |

# Firmware Version 1.00.0F

Date:  2015/07/24

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| VLAN | Fixed when Management VLAN port only trunk mode , after output config file it will lose Management port setting. |

# Firmware Version 1.00.0E

Date:  2015/07/22

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Telnet | Fixed Telnet timeout cannot set to disable. |
| Config restore | When device restore a config file with incorrect VLAN command content. It will cause device restore process fail. |
| SNMP | Fixed when receive get-next-request OID is 1.3.6.1.2.1.17.4.2.0(dot1dTp aging time) will cause SNMP service disable.- |

# Firmware Version 1.00.0D

Date:  2015/07/17

| New Feature / System Release | Description |
| --- | --- |
| System | New add show CPU utilization and memory usage rate function. CLI : Switch# show switch-info cpu-mem-statistice |
| Update | Description |
| Loop detection | Behavior change : 1.Looped port from “port shutdown” change to listening 2.Looped port LED will blink at a cycle of 2-second period (Orange light flash) 3.Unblock looped port situation under the looped port not receive loop detection packet. |
| Telnet | CLI can support maximum 4 sessions at the same time |
| SNMP | Increase device response SNMP port status and counter speed. |
| Port isolation | Separate Port based VLAN and VLAN isolation config |
| DHCP option 82 | Fixed CLI cannot enter 63 characters in DHCP option 82 remote ID and circuit ID |
| IGMP/MLD snooping | Fixed IGMP/MLD VLAN ID cannot enable “snooping” and “Querying” function |

# Firmware Version 1.00.0C

Date:  2015/06/05

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | Fixed after restore config then reboot , device will change to default setting |
| SFP port | When plug-in 100M UTP (RJ-45) type SFP, the link LED shows link-up (light) without connected UTP cable. |
| SNMP | Power down trap source IP are wrong |

# Firmware Version 1.00.0B

Date:  2015/05/26

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | When save config will lead to CLI can not access. |

# Firmware Version 1.00.0A

Date:  2015/05/04

| New Feature / System Release | Description |
| --- | --- |
| Firmware upgrade | WEB UI support HTTP firmware upgrade and config restore. |
| Update | Description |
| SNMP | Speed up show MIB counter. |
| SNMP | Fixed device not sent “cold-start” trap. |

# Firmware Version 1.00.09

Date:  2015/05/04

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| MAC address table | Improve MAC address aging time can config up to 172800 seconds |
| System information | Improve FAN status display on show system information. |
| DHCP | Fixed device boot up and send DHCP DISCOVER packet before port link up. |
| System | Fixed when SFP slot insert Copper port SFP without link up , port of LED will show green light. |
| MAC address | Fixed show MAC address table by port will cause error. |
| SNMP | Fixed SNMP will send battery “Charge “ , “Discharge” trap twice. |

# Firmware Version 1.00.08

Date:  2015/3/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| DHCP auto-provision | Fixed after device load to default factory setting , DHCP auto-provision cannot success update config file |

# Firmware Version 1.00.07

Date:  2015/3/30

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| SNTP | Fixed show SNTP status typo “STNP” |
| SNMP | Fixed SNMP statistics polling interval time cannot set to 60 seconds. |

# Firmware Version 1.00.06

Date:  2015/3/23

| New Feature / System Release | Description |
| --- | --- |
| DHCP Snooping | Add DHCP option82 circuit and remote ID value can be manual configuration. |
| Update | Description |
| WEB UI | Fixed MAC address table no shows update information when press “Next” or “Top” button twice. |
| WEB UI | Fixed Port 25-28 when do port shutdown still shows link up status |
| SNMP | After device restore config and reboot , the SNMP can not modify any setting. |
| Default gateway | Allow default gateway IP can configurable to 0.0.0.0 IP address. |
| SNMP | Fixed SNMP allow config port under 1000M speed can working on half duplex mode. |
| Loop detection | Fixed loop detection did not correct block loop port. |
| LLDP | Fixed LLD system-name will lose config setting after device reboot. |
| LLDP | Fixed When config LLDP system-name over 55 characters it will overwrite LLDP system-description config characters. |
| Q-in-Q | Fixed Q-in-Q ISP port will not add S-tag under following situation. a.Q-in-Q client port enable DHCP snooping receive DHCP packet. b. IGMP control packet outgoing from Q-in-Q ISP port snooping enable when outgoing from Q-in-Q ISP port IGMP control packet |
| IGMP snooping | Fixed IGMP group table show empty value, when it receive IGMP join packet that out of 224~239.x.x.x multicast IP rage. |
| IGMP snooping | Fixed IGMP group table cannot remove 227.1.1.10 multicast group when it does not receive any IGMP join anymore. |
| Port counter | Modified port counter oversize packet as ‘over Max byte’. |

# Firmware Version 1.00.05

Date:  2015/3/12

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | Fixed when show 802.1x port status via SNMP , it will cause device slowly response SNMP polling and CLI malfunction. |
| WEB UI | Fixed ACL configuration page in WEB UI without delete button. |
| WEB UI | Improve ACL configuration page in WEB UI display. |
| IPv6 | Solicit and Request not included DHCP vendor ID in DHCPv6 option on default value , but after change ID will include. |
| ACL | Improve ACL rule port deny ingress and permit behavior. |
| LLDP | Fixed LLDP cannot shows IPv6 address. |

# Firmware Version 1.00.04

Date:  2015/3/07

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| Layer 2 BPDU packet filter | Fixed when enable layer 2 BPDU packets filter cannot filter LLDP packets. |
| Q-in-Q | Allow Q-in-Q ISP port can pass through Single Tag VLAN traffic. |
| IGMP snooping | Fixed When enable multiple VLAN on IGMP groups , it will cause snooping client cannot received IGMP query. |

# Firmware Version 1.00.03

Date:  2015/3/07

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| WEB UI | Improve WEB UI Switch panel renew frequency. |

# Firmware Version 1.00.02

Date:  2015/3/05

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| System | Fixed when IGMP snooping features enable for a period of time , device will automatically reboot. |
| IP source guard | Fixed when IP source guard enable and config to dhcp mode, it will cause the clients cannot get IPv6 address from DHCP server. |

# Firmware Version 1.00.01

Date:  2015/1/23

| New Feature / System Release | Description |
| --- | --- |
| N/A | N/A |
| Update | Description |
| N/A | N/A |
| RSTP | Reduce system resource usage when enable RSTP feature. |
| Packet forward | Fixed DHCPv6 cannot send SOLICIT packet. |
| IPv6 | Fixed IPv6 global address will invalid when save config and reboot. |